Robust, fast active spectral sensing via orthogonal temporal-frequency encoding

Introduction

Spectroscopic sensing has emerged as a cornerstone technology across diverse disciplines, finding critical utility in precision agriculture, environmental monitoring, and clinical diagnostics. However, achieving high-fidelity spectroscopy in real-world environments remains a significant challenge, primarily due to unpredictable ambient interference. Conventional dispersive spectrometers based on gratings or prisms are not only compromised by bulky form factors and high costs but also inherently constrained by the need for narrow entrance slits, which severely limits optical throughput. More critically, these systems indiscriminately accumulate total photon flux due to a lack of intrinsic mechanisms to decouple the target signal from background illumination. This mixes the target signal with ambient interference, causing spectral reconstruction to fail in real-world scenarios.

Computational spectroscopy has emerged as a promising pathway to overcome these physical limitations. However, existing paradigms remain constrained by an inherent trade-off between signal-to-noise ratio (SNR), acquisition speed, and spectral resolution. Snapshot approaches, such as filter arrays, achieve acquisition speed but inherently suffer from a spatial-spectral trade-off that limits their spectral resolution. Conversely, time-division multiplexing (TDM) approaches, ranging from tunable spectral filters to sequential active illumination, offer high spectral fidelity but rely on serial acquisition. This temporal sequentiality dictates that the total acquisition time scales linearly with the number of spectral channels (Ttotal∝N), rendering them ineffective for capturing transient dynamics. Ultimately, these computational frameworks lack an intrinsic encoding mechanism to effectively decouple the active signal from ambient interference. Consequently, they remain strictly bound to controlled lighting environments.

In this work, we propose orthogonal time-frequency encoding (OTFE) to address the challenge of retrieving the encoded spectral response amidst complex environmental lighting. Rather than attempting to disentangle mixed signals post-detection, our strategy ensures rigorous isolation by encoding each spectral basis function via a distinct orthogonal modulation kernel at the illumination source. This mechanism fundamentally shifts spectral acquisition from a serial time-domain sequence to a parallelized frequency-domain architecture. Consequently, it eliminates the need for sequential illumination, enabling the simultaneous reconstruction of all encoded spectral channels via a single-pixel detector in a single-shot acquisition.

The resulting demodulation process leverages orthogonality-based filtering, efficiently rejecting all ambient light and out-of-band interference that does not match the specific modulation frequencies. We demonstrate that this strategy enables accurate spectral reconstruction even under a SNR as low as -11.77 dB—a regime where conventional approaches fail. Furthermore, it achieves microsecond-level sensing with acquisition times as short as 250 μs. This work establishes a robust, fast and low-cost framework for spectral sensing in challenging environments.

Principle

Under the TDM architecture, spectral channels are activated sequentially according to a predefined timing sequence. The instantaneous intensity yTDMt received by the detector at any given time t represents the superposition of the spectral signal from the single active channel within the current time slot and the ambient noise:

|  | yTDMt=λ1λ2k=1KPkλ⋅rectt-kττ⋅RλSλ dλ+Inoiset | (1) |
| --- | --- | --- |

where Pkλ denotes the spectral basis function of the k-th channel at the illumination source. rect⋅ represents the unit rectangular function, acting as a temporal gating operator that characterizes the serial scanning mechanism, where the k-th channel is active solely during its specific integration time τ. Rλ represents the reflectance or transmittance spectrum of the target, Sλ is the spectral responsivity of the detector, which characterizes overall efficiency of the optical-to-electrical conversion determined collectively by the photoelectric conversion efficiency and the transimpedance gain. And Inoiset denotes the ambient light noise.

To transcend the serial constraints of TDM, we fundamentally alter the encoding mechanism for each spectral channel indexed by k (k=1, …, K). Specifically, we substitute the binary temporal gating operator rect⋅ with a modulation kernel mkt;fk uniquely assigned to that channel. This substitution eliminates the requirement for sequential time slots, allowing all channels to operate in parallel. The modulation kernel for the k-th channel is formulated as:

|  | mkt;fk=ak,0+ak,1cos2πfkt+φk | (2) |
| --- | --- | --- |

where fk is the distinct carrier frequency assigned to channel k. ak,0 represents the DC bias required to ensure non-negative optical intensity. ak,1 denotes the carrier amplitude, and φk is the initial phase of the signal.

Upon simultaneous projection of all spectral channels modulated by their respective kernels, the detector integrates the total photon flux. The received OTFE signal yOTFEt is a composite waveform containing information from all K channels concurrently:

|  | yOTFEt=k=1Kλ1λ2PkλRλSλdλ zk⋅mkt;fk+Inoiset | (3) |
| --- | --- | --- |

Here, to simplify the physical interpretation, we define the scalar zk as the spectral projection value for the k-th channel. Substituting the modulation kernel into equation (3), the received signal yOTFEt can be reorganized into two distinct components:

|  | yOTFEt=k=1Kzkak,0+InoisetDC & Noise component+k=1Kzkak,1cos2πfkt+φkSignal component | (4) |
| --- | --- | --- |

The signal decomposition is achieved by mapping the time-domain observation into the frequency domain. Defining F{⋅} as the continuous Fourier transform operator, the frequency-domain response Yf is given by:

|  | Yf=F{yOTFEt}+k=1K12zkak,1ejφkδf-fk+e-jφkδf+fk | (5) |
| --- | --- | --- |

where δ⋅ denotes the Dirac delta function. This formulation demonstrates the mechanism of noise rejection: as long as the modulation frequencies {fk} are designed to avoid the dominant bands of environmental noise and are mutually non-interfering, the projection value zk can be retrieved precisely by extracting the magnitudes at the corresponding carrier frequencies:

|  | Yfk=12zkak,1 ⇒ zk=2Yfkak,1 | (6) |
| --- | --- | --- |

Once the projection values for all K channels are retrieved, they are aggregated to form the measurement vector z=z1,z2,…,zKT.To recover the spectrum Rλ of the target, we discretize the spectral range λ1,λ2 into N bands and model the measurement process as a system of linear equations:

|  | z=Wr+e | (7) |
| --- | --- | --- |

where r∈RN represents the ground truth (GT) spectrum, and W∈RK×N is the pre-calibrated system matrix determined by the source emission spectra and detector responsivity. The term e accounts for the system noise and measurement residuals. Consequently, the spectral reconstruction is treated as an inverse problem. The estimated spectrum r is obtained by finding the solution that minimizes the reconstruction error:

|  | r=argminrz-Wr22 | (8) |
| --- | --- | --- |

Experiment

To empirically validate the proposed OTFE framework, we develop a spectral sensing prototype. OTFE is methodologically agnostic to the specific light source, provided it supports high-speed temporal modulation and possesses spectral diversity. In this study, we select LEDs as the source due to their narrowband spectral diversity and MHz-level modulation bandwidth for efficient orthogonal encoding.

Correspondingly, the detection module is designed to match this high-frequency modulation capability. While standard 2D image sensors (CMOS/CCD) provide spatial resolution, their limited frame rates (typically < 100 fps) constitute a fundamental bottleneck for sampling the high-frequency carriers essential for OTFE’s noise rejection mechanism. To fully exploit the SNR advantage—specifically to shift signals beyond the $1/f$ noise corner—we employed a high-bandwidth single-point detector (SPD) rather than a focal plane array. This configuration prioritizes temporal resolution and spectral precision, serving as a rigorous validation platform where the core encoding-decoding physics can be evaluated without the bandwidth limitations of conventional imagers.

As a proof of concept, we built a low-cost system based on LED array to validate the principle. The experimental setup, illustrated in Fig. 1(a), consists of an LED array and a corresponding driver circuit board (Fig. 1 (b)). The array includes 15 LEDs with distinct emission wavelengths, where only 12 are utilized in the experiment to span the visible spectrum from 400 nm to 700 nm. The individual emission spectra of these 12 LEDs and their pairwise correlation coefficients are presented in Fig. 1 (c) and Fig. 1 (d), respectively.

Fig. 1. Spectral characteristics of our system. (a) A photograph of the 15-channel LED array light source (12 LEDs are utilized in the experiment). (b) The corresponding driver circuit board. (c) The emission spectra of the 12 distinct LEDs used in the experiment. (d) A heatmap of the correlation coefficients between the LED spectra, with an average correlation coefficient of 0.1943.

To enable independent intensity control, each LED is managed by a dedicated driver circuit featuring a UMW PT4115 chip. The output current, Iout, delivered to each LED is linearly controlled by the duty cycle, D (0% to 100%), of a PWM signal according to the formula:

Iout=0.1DRS,#(11)

where RS is a fixed internal resistance. The requisite PWM signals are generated by STM32F103C8T6 microcontrollers, which serve as the central controller for the system.

The use of square wave signals for PWM driving necessitates careful selection of the modulation frequencies to prevent inter-channel interference. Specifically, if the frequency of one channel is an integer multiple of another, its high-order harmonics could alias with the fundamental frequency of the other channel, leading to signal crosstalk and beating phenomena. To prevent such interference, we assign a unique prime number as the fundamental modulation frequency for each of the 12 LED channels.

The frequency selection strategy governs the data acquisition parameters required for successful demodulation. The frequency resolution, Δf, of the FFT is the reciprocal of the total sampling duration, T:

Δf=1T.#12

Therefore, to reliably distinguish between the closely spaced carrier frequencies employed in our FDM scheme, a sufficiently fine frequency resolution is essential. Accordingly, we set the total acquisition time to 1.0 second, yielding a frequency resolution of 1.0 Hz, which provides an adequate margin for resolving all channels without ambiguity. In the practical scenario, the acquisition time can be decreased if the modulation frequency interval between each LED is large enough.

Fig. 2. Experimental results for spectral reconstruction. (a) The ground truth color images of a standard 24-patch colorchecker. (b) The reconstructed color images with high visual fidelity to the ground truth. (c) Quantitative comparison of reconstructed spectra (solid lines with circles) and ground truth spectra (dashed lines) for 12 selected color patches. (d) The system's performance in reconstructing narrow-band spectra, where the dashed lines represent the ground truth spectra and the solid lines with circles represent the reconstructed spectra.

In the experiment, our LED array illuminates a standard 24-patch colorchecker, and the reflected optical signals are captured by a silicon PD (Thorlabs PDA100A2) and digitized using an oscilloscope (SIGLENT SDS2202X Plus). The results of spectral reconstruction are presented in Fig. 2. A qualitative comparison between the ground truth color images (Fig. 2(a)) and the reconstructed images (Fig. 2(b)) reveals a high degree of visual fidelity. Fig. 2(c) presents the detailed spectral data for 12 selected patches. The reconstructed spectra show high agreement with the ground truth spectra, confirming the system's ability to accurately recover a variety of broadband spectral signals. To test the system's ability to resolve narrowband features, we measure a series of narrowband input signals. As shown in Fig. 2(d), the reconstructed spectra accurately capture the sharp peak of the ground truth spectra, confirming that the system capably resolves fine spectral features.

Fig. 3. Experimental results demonstrating the system's robustness against ambient noise. (a) Time-domain plots of the pure signal (top), strong external noise (middle), and the combined signal with an signal-to-noise ratio (SNR) of -15.46 dB (bottom). (b) The top panel clearly shows the 12 peaks corresponding to the unique modulation frequencies of the modulated LEDs, namely, 113, 127, 131, 137, 139, 149, 151, 157, 163, 167, 173, 179 Hz. The desired signal peaks remain clearly distinguishable from the large noise peak in the frequency domain, as shown in the bottom panel. (c) The spectral reconstruction results of a representative signal under various SNRs in experiments. Details on the calculation of SNR are provided in Supplement Information. (d) Quantitative analysis of the reconstruction's root mean square error (RMSE) as a function of SNR. The red dashed line indicates the baseline RMSE for a noiseless reconstruction, demonstrating that the reconstruction error for a case with an SNR of -15.46 dB remains comparable to that for the noise-free case.

To evaluate the system's robustness, we test its performance in an environment containing two distinct types of optical interference: a constant, unmodulated background light source, which produces a direct current (DC) offset, and a separate interfering light periodically modulated at around 247 Hz. As shown in Fig. 3(a), the desired signal is obscured by this combined interference in the time domain, resulting in an SNR of -15.46 dB. The FDM approach is inherently effective against such interference. The modulation places the signal channels into a frequency band that is well separated from both the DC component and the dominant alternating current (AC) noise peak, allowing the 12 signal peaks to be clearly isolated (Fig. 3(b)).

This noise immunity is quantified by the reconstruction performance shown in Fig. 3 (c-d). The RMSE remains remarkably low and stable across a wide range of SNRs. The reconstruction accuracy at the challenging -15.46 dB SNR is comparable to the noise-free baseline (Fig. 3 (d)). This ability to maintain performance even when the signal is significantly weaker than the interference validates the robustness and noise-rejection capability of our proposed spectroscopy, confirming its suitability for real-world applications.

Conclusion and discussion.

In this work, we have proposed and demonstrated a new computational spectroscopy based on temporally-coded parallel acquisition. By assigning a unique temporal code mkt to each spectral basis Lk of an active illumination system, our method successfully encodes the complete all spectral information onto a single-pixel detector within a single acquisition. This approach maintains the hardware simplicity of active systems while achieving the parallel information capture capability previously limited to traditional passive systems. Experimental results validate that it can achieve reconstruction of both broadband and narrow-band spectra. The system also demonstrates exceptional immunity to ambient noise. This robustness is an inherent consequence of the orthogonal decomposition process (Eq. 6-7), which innately rejects any signal—including DC ambient light and periodic interference—that does not match the predefined temporal codes. This allows for robust spectral recovery even in challenging, low-SNR environments (-15.46 dB).

This work can be extended in several promising directions. The 1.0-second acquisition time in our experiment is a direct consequence of the 1.0 Hz frequency resolution (Δf) chosen to separate our set of temporal codes. The theoretical speed limit of this architecture is not determined by the number of sources K, but by the minimum frequency separation (Δfmin) between the codes, as Ttotal≥1/Δfmin. By designing a set of temporal codes with a larger Δfmin (e.g., in the kHz range), the acquisition time Ttotal could be reduced to milliseconds or even microseconds, as a coarser frequency resolution would suffice. Given that active sources (e.g., laser diodes) can be modulated at gigahertz rates, this technique holds great potential for ultrafast spectroscopy. Furthermore, the spectral range could be expanded to the ultraviolet or infrared regions by incorporating appropriate active sources and detectors. Increasing the number of spectral bases K would improve the conditioning of the inverse problem (Eq. 10), further enhancing reconstruction fidelity. Finally, the system's active, code-based nature opens possibilities for real-time adaptive sensing, where the temporal codes mkt could be dynamically adjusted to optimize the reconstruction for specific spectral features of interest.