Robust computational spectral sensing via orthogonal frequency encoding

Yijia Zeng1, Zhi Zhang1, Quanzhou Long1, Yafan Jin1, Zijian Lin1, Feng Zhou1, Tingbiao Guo1,2,*, Sailing He1,2,3,*

1 Centre for Optical and Electromagnetic Research, National Engineering Research Center for Optical Instruments, College of Optical Science and Engineering, Zhejiang University, Hangzhou, Zhejiang, China

2 Institute of Fundamental and Transdisciplinary Research, Zhejiang University, Hangzhou, Zhejiang, China

3 Department of Electromagnetic Engineering, School of Electrical Engineering, Royal Institute of Technology, Stockholm S-10044, Sweden

*Corresponding authors: sailing@kth.se and tbguo@zju.edu.cn

Abstract

High-fidelity spectroscopy in real-world environments is limited by ambient interference and the inherent trade-off between acquisition speed and signal-to-noise ratio (SNR). Conventional time-division multiplexing approaches often fail as they lack intrinsic mechanisms to decouple target signals from background illumination. Here, we propose orthogonal frequency encoding, a robust framework that shifts spectral acquisition from serial scanning to a parallelized frequency-domain architecture. By encoding each spectral channel with a distinct orthogonal frequency kernel, it allocates carriers to frequency gaps to evade structured environmental noise. As proof of concept, we use a programmable LED array to demonstrate scan-free computational spectral reconstruction based on this framework. The system exhibits immunity to high-energy interference, maintaining high reconstruction fidelity (RMSE ≈ 0.0261) under a modulation SNR of -11.77 dB. This work establishes a fast, low-cost strategy for spectral sensing in challenging environments.

Introduction

Spectroscopic sensing is an essential technology across diverse disciplines[1], with applications in precision agriculture[2], environmental monitoring[3], and clinical diagnostics[4]. However, achieving high-fidelity spectroscopy in real-world environments remains a challenge due to ambient interference. Conventional dispersive spectrometers based on gratings or prisms are not only compromised by bulky form factors and high costs but also inherently constrained by the need for narrow entrance slits, which limits optical throughput[5]. Moreover, these systems indiscriminately accumulate total photon flux due to a lack of intrinsic mechanisms to decouple the target signal from background illumination.[6] This mixes the target signal with ambient interference, causing spectral reconstruction to fail in real-world scenarios.

Computational spectroscopy can overcome these physical limitations[7]. Existing schemes remain constrained by an inherent trade-off between signal-to-noise ratio (SNR)[8], acquisition speed[9], and spectral resolution[10]. Snapshot approaches[11], such as filter arrays, achieve high acquisition speed but suffer from a spatial-spectral trade-off that limits their spectral resolution[12]. Conversely, time division multiplexing (TDM) methods rely on sequential scanning, trading off temporal resolution for spectral accuracy. This sequentially dictates that the total acquisition time scales linearly with the number of spectral channels (Ttotal∝N), rendering them ineffective for capturing high-speed dynamics[13]. Moreover, these computational frameworks remain bound to controlled lighting environments due to their lack of an intrinsic encoding mechanism to decouple the active signal from ambient interference.

In this work, we propose a robust computational framework for spectral sensing in challenging environments, termed orthogonal frequency encoding (OFE). Unlike standard lock-in detection or post-detection unmixing strategies[14], this scheme ensures isolation by encoding each spectral basis function via a distinct orthogonal frequency kernel, enabling the simultaneous reconstruction of all encoded spectral channels on a single detector within one integration window, without any temporal scanning. In contrast to sequential lock-in detection, which demodulates one spectral component at a time, OFE performs parallel multi-carrier demodulation so that every channel contributes to the observation simultaneously. We employ a low-cost, programmable LED array as a demonstration of this framework. The results demonstrate that this strategy enables high-fidelity spectral reconstruction at a modulation SNR of -11.77 dB. All SNR values reported in this work refer to the modulation SNR defined in Supplement Section 1, i.e., the ratio of carrier-bin power to in-band residual power evaluated at the output of the OFE matched-filter demodulator. Furthermore, it achieves fast sensing with acquisition times as short as 250 μs. These results validate OFE as a practical approach for spectral sensing under adverse conditions.

Principle

A generalized spectral sensing architecture can be abstracted into two fundamental components: a spectral modulator and a detector. Under a TDM scheme, spectral channels are activated sequentially according to a predefined timing sequence. The instantaneous intensity yTDMt received by the detector at any given time t represents the superposition of the spectral signal from the single channel within the current time slot and the ambient noise:

|  | yTDMt=k=1Kλ1λ2PkλRλSλdλ⋅rectt-kττ+Inoiset | (1) |
| --- | --- | --- |

where Pkλ denotes the spectral basis function of the k-th channel for the spectral sensing system. rect⋅ represents the unit rectangular function, acting as a temporal gating operator that characterizes the serial scanning mechanism, where the k-th channel is active solely during its specific integration time τ. Rλ represents the reflectance or transmittance spectrum of the target, Sλ denotes the detector’s spectral responsivity, encompassing both photoelectric conversion efficiency and transimpedance gain, Inoiset denotes the ambient light noise.

In the orthogonal frequency encoding scheme, we substitute the binary temporal gating operator rect⋅ with a modulation kernel mkt;fk uniquely assigned to the k-th channel. This substitution eliminates the requirement for sequential time slots, allowing all channels to operate in parallel. The modulation kernel for the k-th channel is formulated as:

|  | mkt;fk=ak,0+ak,1cos2πfkt+φk | (2) |
| --- | --- | --- |

where fk is the distinct carrier frequency assigned to channel k. ak,0 represents the direct current (DC) bias required to ensure non-negative optical intensity. ak,1 denotes the carrier amplitude, and φk is the initial phase of the signal.

Upon simultaneous collection of all spectral channels modulated by their respective kernels, the detector integrates the total photon flux. The received OFE signal yOFEt is a composite waveform containing information from all K channels concurrently:

|  | yOFEt=k=1Kλ1λ2PkλRλSλdλzk⋅mkt;fk+Inoiset | (3) |
| --- | --- | --- |

To simplify the physical interpretation, we define the scalar zk as the spectral projection value for the k-th channel. Substituting the modulation kernel into equation (3), the received signal yOFEt can be reorganized into two distinct components:

|  | yOFEt=k=1Kzkak,0+InoisetDC & Noise component+k=1Kzkak,1cos2πfkt+φkSignal component | (4) |
| --- | --- | --- |

The signal decomposition is achieved by mapping the time-domain observation into the frequency domain. Defining F{⋅} as the continuous Fourier transform operator, the frequency-domain response Yf is given by:

|  | Yf=F{Inoiset}+k=1K12zkak,1ejφkδf-fk+e-jφkδf+fk | (5) |
| --- | --- | --- |

where δ⋅ denotes the Dirac delta function. This formulation demonstrates the mechanism of noise rejection: if the modulation frequencies {fk} are designed to avoid the bands of environmental noise, the projection value zk is retrieved by extracting the magnitudes at the corresponding carrier frequencies:

|  | Yfk=12zkak,1⇒zk=2Yfkak,1 | (6) |
| --- | --- | --- |

To ensure orthogonality within a physical optoelectronic system, the carrier allocation must satisfy three boundary conditions: spectral separability (fi-fj>Δf,   where Δf is the frequency resolution), harmonic interference avoidance (fj≠n⋅fi), and intermodulation avoidance (fi±fj≠fk). Mathematical derivations of these constraints and the corresponding switch search algorithm are detailed in Supplement Sections 2 and 3.

Figure 1. OFE schematic diagram. Each spectra basis function undergoes modulation via a distinct orthogonal kernel. These components are linearly superimposed to form a composite time-domain signal, which is contaminated by additive noise during physical transmission. Upon reception by a sensor, a transform (e.g., Fourier transform) maps the temporal waveform into a frequency-domain signal, isolating the deterministic carrier peaks from the noise floor. Finally, a computational algorithm is used to reconstruct the target spectra based on the spectra basis and carrier peaks.

OFE possesses two advantages over traditional TDM: the multiplexing energy gain and orthogonal interference immunity. The multiplexing advantage stems from its parallel collection over K channels. Coherent frequency-domain demodulation acts as a matched filter, bounding the noise variance to σOFE2∝1/K. This inherent variance ratio of K dictates an enhancement in the signal-to-noise ratio (SNR) by a factor of K under identical acquisition time. This theoretical multiplexing gain, conceptually analogous to Fellgett’s advantage, preserves high-fidelity reconstruction in additive white Gaussian noise (AWGN) environments.√K

Beyond multiplexing advantage, the orthogonal interference immunity addresses TDM’s vulnerability to unselective noise integration. Traditional time-domain amplitude measurements accumulate all additive noise components spanning the detector’s bandwidth. OFE encodes spectral information into specific discrete orthogonal subcarriers. During coherent demodulation, any external interference is rejected from the assigned carriers exceeds the system’s frequency resolution limit (Δf=1/Ts). This frequency-domain encoding enables the excision of all out-of-band noise, providing the system with immunity against complex time-domain distortions.

Once the projection values for all K channels are retrieved, they are aggregated to form the measurement vector z=z1,z2,…,zKT. To recover the spectrum Rλ of the target, we discretize the spectral range λ1,λ2 into N bands and model the measurement process as a system of linear equations:

|  | z=Wr+e | (7) |
| --- | --- | --- |

where r∈RN represents the ground truth (GT) spectrum, and W∈RK×N is the pre-calibrated spectral basis matrix determined by the spectral sensing system. The term e accounts for the system noise and measurement residuals. Consequently, spectral reconstruction is an ill-posed inverse problem. We describe the reconstruction as a constrained convex optimization problem and employ a projected gradient descent (PGD) algorithm to minimize a composite objective function that incorporates both physical boundary constraints and adaptive spectral priors:

|  | r=min0⪯r⪯112z-Wr22+Rr | (8) |
| --- | --- | --- |

where Rr is a generalized regularization term empirically configured based on target spectral topology. For targets with distinct narrowband features, we apply a sparsity prior to resolve fine spectral structures, defining Rr=β1r. For targets characterized by continuous, broadband spectra, we enforce smoothness and structural continuity by defining Rr=β22r22+β32Dr22, where D∈RN-1×N is the first-order difference operator. Detailed hyperparameter configurations and the specific algorithmic implementation are cataloged in Supplement Section 4.

Experiment

To isolate and quantify the multiplexing advantage of the OFE framework against AWGN, we establish a simulation benchmark. Operating across the visible spectrum (400–700 nm), the architecture deploys K=12 discrete spectral channels (detailed in Supplement Section 5, Fig. S1). The spectral basis functions are instantiated from the empirically calibrated radiometric profiles of LED array in our lab (detailed in Supplement Section 6, Fig. S2). This ensures that the system matrix W encapsulates the non-ideal spectral irregularities and native radiometric variations inherent to the physical hardware. In the time domain, these channels are modulated by the orthogonal composite sinusoidal carriers. The spectral reconstruction is conducted utilizing the CAVE multispectral database[15].

To ensure baseline orthogonality without introducing hardware-specific combinatorial complexities, we employ a straightforward sub-octave linearly spaced carrier set for all numerical TDM-vs-OFE comparative simulations. Specifically, this frequency set spans from fmin=380 kHz to fmax=490 kHz with a uniform frequency internal of 10 kHz (we term this frequency selection as Configuration 1, and detailed algorithmic implementation is provided in Supplement Section 3). By the condition of orthogonal integration (T = 1/Δf), this 10 kHz spectral resolution dictates a 100 μs acquisition window. To ensure an equivalent temporal and energy budget for comparison, the TDM benchmark uniformly partitions the identical 100 μs acquisition window into K=12 sequential time slots (≈8.33 μs per channel), with each channel operating at a constant direct current (DC) baseline. To mitigate computational redundancy while preserving statistical robustness, the errors reported in Figure 2 are evaluated over a 10% globally random subset of pixels from all 29 images in the CAVE dataset.

Figure 2. Numerical benchmarking of the OFE and TDM frameworks. (a) Time-domain waveforms with AWGN (modulation SNR = 25 dB). (b) Spectral reconstruction performance: RMSE (top) and demodulation error (bottom) versus modulation SNR. Data points represent statistical averages across the CAVE database. (c) Time-domain waveforms under multitone interference (Δfguard=10 kHz, modulation SNR = -10 dB). The 10 kHz offset corresponds to the 1/T frequency resolution limit of the sampling window. (d) RMSE versus modulation SNR under multitone interference at varied frequency offsets. TDM yields a single baseline (grey dashed line) due to the absence of frequency-domain selectivity. OFE achieves interference rejection exclusively when Δfguard meets the 10 kHz integer-cycle threshold.

Figure 2(a) illustrates the time-domain waveforms of both schemes under AWGN (modulation SNR = 25 dB) within a 20 μs window. Note that a DC bias (ak,0) is added to the OFE waveform, presenting the zero-mean information-bearing composite signal. The OFE signal exhibits a continuous, dense composite waveform characteristic of parallel frequency multiplexing, whereas TDM presents discrete, sequential step-like features. Figure 2(b) quantifies the reconstruction accuracy across a broad modulation SNR range (Supplement Section 1). To evaluate the spectral reconstruction, we quantify the system’s performance at two stages: the intermediate demodulation error (defined as the relative Euclidean distance z-z2/z2 between the extracted and ground truth spectral projections) and the final spectral reconstruction RMSE (Fig. 2b). At the demodulation stage, OFE demonstrates superior projection fidelity across all evaluated noise levels. This stability propagates to the final spectral reconstruction. The observed separation between the OFE and TDM RMSE curves align with the theoretical K (approximately 3.46-fold for K=12) multiplexing energy gain derived in the previous section.

Beyond broadband AWGN, practical systems frequently encounter structured interference. To isolate the system’s dynamic response to such artifacts, we maintain the identical TDM and OFE configurations used in the AWGN benchmark, but substitute the broadband noise model with multitone interference (Supplement Section 7) operating at a modulation SNR of -10 dB. We parameterize the frequency offset between each interfering tone and its adjacent OFE carrier as the guard band, Δfguard=fnoise-fcarrier.

Figures 2(c) and 2(d) demonstrate the interference rejection mechanism. In the time-domain visualization (Fig. 2c), multitone interference (modulation SNR = -10 dB) at an offset of Δfguard=10 kHz induces high-frequency ripples. Because this offset matches the frequency resolution limit (1/T) of the acquisition window, the noise harmonic completes integer cycles within the integration period. This satisfies the orthogonality condition, enabling its elimination during frequency-domain demodulation. Figure 2(d) quantifies this threshold behavior. TDM lacks frequency-domain selectivity, resulting in a uniform degradation of its time-domain integration across all evaluated offsets and a collapse into a single baseline. For OFE, noise immunity is governed by the 1/T (1/100 μs) threshold. Offsets below this limit (Δfguard=1 and 5 kHz) fail to satisfy the coherent integration constraint, resulting in non-zero residual integrals that manifest as spectral leakage and elevated RMSE. Once the offset reaches the 10 kHz threshold, orthogonality is satisfied, yielding a low reconstruction error independent of the interference intensity.

Furthermore, to validate the framework’s robustness against timing jitters and hardware synchronization constraints, we evaluate its sensitivity to phase noise (detailed in Supplement Section 8). Simulations demonstrate that the reconstruction fidelity remains stable for phase perturbation within the 0.1 rad regime.

Following the simulation-based verification of the noise immunity, we develop a prototype to validate the OFE framework experimentally. To hardware-instantiate the K=12 spectral channels modeled in the numerical benchmark, we deploy the aforementioned programmable LED array as the physical spectral encoder (detailed in Supplement Section 6, Fig. S2). After the encoded illumination acquires the target’s spectral signature, a photodiode coupled to a high-speed oscilloscope captures the resulting time-domain composite waveform. Digital demodulation is executed via the fast Fourier transform (FFT), mapping the temporal signal into the frequency domain to extract the orthogonal spectral projections. Figure 3(a) schematizes the architectural configurations for both reflective and transmissive modalities. Hardware specifications, including the physical bandwidth constraints, alongside the benchtop photographs are detailed in Supplement Section 9, Fig. S4.

To assess the baseline reconstruction capabilities without external electrical interference, we configure the hardware for a high-speed acquisition (Ts=250 μs). As established by the empirical benchmark (Fig. S5 in Supplement Section 10), this dictates a theoretical frequency resolution of Δf = 4 kHz. To multiplex all K=12 carriers within the available hardware bandwidth under this constraint, we deploy a greedy search algorithm to optimize the frequency carrier selection (Configuration 2). Unlike the dense, linearly spaced configuration (Configuration 1) used in previous simulation, this strategy yields a non-uniformly distributed carrier set (Hopt∈30,410 kHz) that navigates the intermodulation region, maximizing spectrum utilization in a clean electromagnetic environment (the exact numerical array is detailed in Supplement Section 3).

Figure 3. Experimental validation of the OFE prototype. (a) Schematics of the optical paths for (i) reflective and (ii) transmissive modalities. (b) Reconstructed surface reflectance of the 24-patch reflective colorchecker (odd-numbered patches from #1 to #23, indexed sequentially left-to-right, top-to-bottom). Open circles denote the OFE prediction (Pred.) against the ground truth (GT, dashed lines), yielding an average RMSE of 0.0422. Insets display the corresponding sRGB color renderings. (c) Reconstruction profiles of isolated 10 nm FWHM narrowband spectral peaks. In the experiment, the sampling time Ts is 250 μs, the frequency carriers are selected within Hopt∈30,410 kHz under Configuration 2.

Figure 3(b) demonstrates the recovery of continuous broadband envelopes from a 24-patch reflective colorchecker under reflective mode (Fig. 3a (i)). The system yields a mean RMSE of 0.0422, which translates into consistent sRGB colorimetric reproduction (inset). Figure 3(c) validates the recovery of isolated 10 nm full width at half maximum (FWHM) narrowband filters in transmissive mode (Fig. 3a (ii)). The extracted spectral vectors resolve these sharp peaks, preserving the discontinuous features. K=12

Furthermore, we introduce a 50 kHz switched-mode power supply (SMPS) noise source to evaluate its orthogonal interference immunity advantage with the setup shown in Figure 3a(ii). Patch #15 (red) of a transmissive colorchecker is employed as a representative sample. As depicted in the frequency-domain analysis (Figure 4d), the SMPS injects deterministic harmonics that constitute the primary components spectrum up to approximately 350 kHz. To evade this noise, we shift all OFE carriers, spaced at 10 kHz intervals, into the higher 380–490 kHz band under Configuration 1 (Fig. 4c). As dictated by the empirical benchmark (Fig. S5 in Supplement Section 10), a longer sampling (Ts=1000 μs) is needed to distinguish each frequency carrier.

Figure 4. Experimental evaluation under 50 kHz SMPS interference. Time-domain composite signals under (a) high modulation SNR (23.11 dB) and (b) low modulation SNR (-11.77 dB). (c) Frequency-domain spectrum of the clean OFE signal, with black triangles marking the sub-octave carriers (380–490 kHz). (d) Frequency-domain spectrum of the interference environment, illustrating the physical isolation between the low-frequency SMPS harmonics and the high-frequency OFE signal area. (e) Reconstruction RMSE versus modulation SNR (Supplement Section 1), demonstrating a robustness plateau up to around -12 dB threshold. (f) Reconstructed transmittance spectra across varying noise levels against the ground truth (solid black line). In the experiment, the sampling time Ts is 1000 μs, the frequency carriers are selected in 380–490 kHz band with uniform interval under Configuration 1.

To quantify the system’s environmental resilience, we evaluate the modulation SNR defined in Supplement Section 1 from the measured waveforms. The signal power (Psignal) is defined as the energy at the fundamental carrier frequencies (the energy leaked into higher-order hardware harmonics is regarded as the loss). The noise power (Pnoise) is defined as the residual energy after excising the fundamental carriers and their higher-order harmonic artifacts, thereby isolating the true external environmental interference floor. Figures 4(a) and 4(b) compare the time-domain composite signals under high modulation SNR (23.11 dB) and low modulation SNR (-11.77 dB) conditions. In Figure 4(b), due to the SMPS noise interference, the original signal is distorted with periodic modulation envelope. The system’s resilience is evidenced by the frequency-domain separation depicted in Figures 4(c) and 4(d). Operating under the sub-octave configuration (Configuration 1), the OFE carriers are confined into the 380–490 kHz high-frequency band (Fig. 4c). This structural frequency isolation ensures that despite the time-domain distortion observed in Fig. 4b, the orthogonal carriers remain uncontaminated.

Figure 4(e) quantifies statistical tolerance to this environmental noise. The spectral reconstruction RMSE exhibits a robustness plateau (0.0261) from 23.11 dB to even -11.77 dB, with performance degrade initiating when the modulation SNR breaches around the -12 dB threshold. Figure 4(f) maps the reconstructed spectra across the evaluated noise levels. Down to the -11.77 dB threshold, the extracted curves show minimal deviation from the ground truth (solid black line), validating the framework’s robustness within the sub-noise regime.

Conclusion and discussion

In this work, we propose an orthogonal frequency encoding (OFE) architecture to resolve the speed-SNR trade-off in conventional TDM. By shifting from serial scanning to parallelized frequency-domain encoding, the system can acquire all spectral channels in parallel within a single integration window, eliminating the need for temporal scanning. Unlike standard lock-in techniques[16], OFE employs a strategic frequency allocation scheme that exploits the spectral gaps of structured environmental noise. This mechanism ensures robust retrieval under sub-noise regime, protecting the system from common interference such as SMPS ripples.

The performance boundaries of OFE are dictated by hardware constraints rather than the encoding strategy itself. The acquisition speed T is constrained by the time-frequency uncertainty principle, T ≈1/Δf. Although the empirical acquisition time of our prototype is bottlenecked at Ts=250 μs, this limitation arises from the switching characteristics of the constant-current LED driver ICs (PT4115), whose modulation bandwidth ceiling is approximately Bmax=500 kHz. The 3-dB optical modulation bandwidth of conventional through-hole LEDs typically reaches several megahertz — well above this driver-imposed limit — indicating that the driver circuit, rather than the LED emitter, constitutes the immediate bottleneck. Upgrading to faster driver topologies would therefore yield a direct improvement in acquisition speed without changing the light source. Beyond conventional LEDs, state-of-the-art micro-LEDs with sub-nanosecond carrier lifetimes can achieve modulation bandwidths exceeding hundreds of megahertz [17], potentially pushing OFE acquisition times into the nanosecond regime and enabling a dramatic increase in the number of spectral channels without compromising temporal resolution. The OFE framework itself poses no intrinsic temporal limits; performance boundaries are dictated entirely by the optoelectronic hardware.

The OFE framework can be extended across spectral, spatial, and frequency dimensions. First, the spectral range is not confined to the visible band. By incorporating appropriate sources, modulators and detectors, the framework can be expanded to the ultraviolet or infrared regions[18]. Second, coupling OFE with event cameras[19] (capable of microsecond-level temporal resolution) or SPAD arrays[20] (achieving picosecond to nanosecond-level precision) facilitates hyperspectral imaging. Third, the active nature of OFE opens new possibilities for real-time adaptive sensing. Unlike passive spectrometers, the spectral channels in OFE can be dynamically optimized based on initial measurements to maximize the reconstruction fidelity for specific spectral features[21], enabling adaptive spectral sensing.

Acknowledgement

We acknowledge the support from the Ningbo Public Welfare Research Program Project (2024Z234), Zhejiang Provincial Major Science and Technology Plan Project（2026LDC01015(JT), the “Pioneer and Leading Goose” R&D Program of Zhejiang (Nos. 2025C02159, 2025C02140, 2025C02159, 2023C03083, 2024C03045, 2024Z146), the National Natural Science Foundation of China (W2412107, 91233208, 62105284), the National Key Research and Development Program of China (2022YFB2804100, 2022YFC3601002), Ningbo Science and Technology Project (Nos. 2020Z077, 2023Z179, 2023Z122, 2024Z146), Science and Technology Plan Key Project of Taizhou City (24gyz01), Ningbo Top Medical and Health Research Program (No.2022020506), the Special Development Fund of Shanghai Zhangjiang Science City, the Special Development Fund of Hangzhou Chengxi Sci-tech Innovation Corridor, and the Fundamental Research Funds for the Central Universities (226-2024-00150). National Ten Thousand Talent Program (Young Talents)

References

- Cui X, Nigmatulin F, Wang L, et al. Miniaturized spectral sensing with a tunable optoelectronic interface[J]. Science advances, 2025, 11(4): eado6886.

- Zhang D, Hou L, Lv L, et al. Precision agriculture: temporal and spatial modeling of wheat canopy spectral characteristics[J]. Agriculture, 2025, 15(3): 326.

- Oberski T, Walendzik B, Szejnfeld M. The monitoring of macroplastic waste in selected environment with UAV and multispectral imaging[J]. Sustainability, 2025, 17(5): 1997.

- Zou X, Shen Y, Jiao C, et al. Label-free resonance Raman spectral imaging reveals magnesium microsphere therapy attenuates oxidative damage in knee osteoarthritic rats[J]. Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 2026: 127743.

- Liu W, Xu S, Lee C. Ultracompact on-chip spectral shaping using pixelated nano-opto-electro-mechanical gratings[J]. Science, 2025, 389(6762): 806-810.

- Grotevent M J, Yakunin S, Bachmann D, et al. Integrated photodetectors for compact Fourier-transform waveguide spectrometers[J]. Nature Photonics, 2023, 17(1): 59-64.

- Hong D, Li C, Yokoya N, et al. Hyperspectral imaging[J]. Nature Reviews Methods Primers, 2026, 6(1): 19.

- Cui K, Huang Y. On-chip spectral imaging and sensing transition towards marketable technologies[J]. Nature Reviews Electrical Engineering, 2025, 2(3): 151-152.

- Chen Z, Qu G, He Z, et al. Single‐Pixel Multispectral Imaging With Customized Pixelated Spectral Mask Plate[J]. Laser & Photonics Reviews, 2026: e03207.

- Hong D, Zhang B, Li X, et al. SpectralGPT: Spectral remote sensing foundation model[J]. IEEE transactions on pattern analysis and machine intelligence, 2024, 46(8): 5227-5244.

- Hua X, Wang Y, Wang S, et al. Ultra-compact snapshot spectral light-field imaging[J]. Nature communications, 2022, 13(1): 2732.

- Yao Z, Liu S, Wang Y, et al. Integrated lithium niobate photonics for sub-ångström snapshot spectroscopy[J]. Nature, 2025, 646(8085): 567-575.

- Zhou P, Lv Y, Zheng Y, et al. CTIS spectral image reconstruction technology based on slit-scanning architecture[J]. Optics Express, 2024, 32(14): 25276-25291.

- Adhikari S, Gross N, Wilson K S, et al. On-chip lock-in detection for ultrafast spectroscopy of single particles[J]. The Journal of Physical Chemistry C, 2024, 128(21): 8708-8715.

- J. I. Park, M. H. Lee, M. D. Grossberg, et al. 2007 IEEE 11th International Conference on Computer Vision. IEEE, 2007: 1-8.

- Shao Z, Huang G, Mielczarek A, et al. Robust spectral sensor for standoff biometric detection[J]. Nature Sensors, 2026: 1-8.

- Wörner H J, Wolf J P. Ultrafast spectroscopy of liquids using extreme-ultraviolet to soft-X-ray pulses[J]. Nature Reviews Chemistry, 2025, 9(3): 185-199.

- Yu H, Memon M H, Yao M, et al. A miniaturized cascaded-diode-array spectral imager[J]. Nature Photonics, 2025, 19(12): 1322-1329.

- Yu B, Liang J, Wang Z, et al. Active hyperspectral imaging using an event camera[C]//Proceedings of the Computer Vision and Pattern Recognition Conference. 2025: 929-939.

- Cao Y, Bao H, Peng L, et al. Single-Photon-Camera-Based Time and Spatially Resolved Electroluminescence Spectroscopy for Micro-LED Analysis[J]. ACS Photonics, 2024, 11(2): 731-736.

- Yuan Y, Sun R, Xie X, et al. Multispectral imaging colorimetric measurement method with illumination-adaptive white calibration[J]. JOSA A, 2025, 42(7): 898-907.