Section 1: Spectral reconstruction based on projected gradient descent

While equation (8) in the main text describes the least-squares solution, solving the ill-posed inverse problem under physical constraints requires adaptive regularization to handle diverse spectral topologies. The L1 norm is utilized to promote sparsity for recovering narrowband peaks, whereas the smoothness term exploits the inherent spectral continuity across adjacent bands to reconstruct broadband signatures; meanwhile, the L2 norm acts as an energy constraint to ensure numerical stability. To solve this, we employ the projected gradient descent (PGD) algorithm to minimize the composite objective function subject to the box constraint C={r∣0⪯r⪯1}:

Jr=min0⪯r⪯112z-Wr22+β1r1+β22r22+β32Dr22

where D∈RN-1×N is the first-order difference matrix:

The gradient ∇Jr is:

∇Jr=WTWr-z+β1sgnr+β2r+β3DTDr

The iterative update rule is given by:

rk+1=minmax0,rk-α⋅∇Jrk,1

Algorithm S1: PGD spectral reconstruction Input: z,W,D, parameters β1,2,3,α,Kmax,τ. Output: r.

- Initialize: r0←0,k ←0, ϵ←∞ - While k<Kmax and ϵ> τ do: - gk←WTWrk-z+β1sgnrk+β2rk+β3DTDrk - rk+1←minmax0,rk-α⋅gk,1 - ϵ←rk+1-rk22 rk22 - k ←k + 1 - End While - Return ·

Section 2: Modulation frequency set selection criteria

Unlike the proposed OFE framework, the conventional time division multiplexing (TDM) scheme requires no complex frequency-domain constraints. In our simulation, TDM utilizes no modulation frequency; instead, it simply divides the 100 μs acquisition window into 12 sequential time slots (≈ 8.33 μs each) of constant DC signal. For the OFE system, which operates over the identical 100 μs acquisition window regardless of its specific frequency configuration, let {f1,f2,…,fK} denote the set of modulation frequencies for the K spectral basis functions. The selection of this frequency set is constrained by both non-ideal hardware characteristics and fundamental signal processing principles. Specifically, it must adhere to the following three physical constraints:

- Spectral separability condition

|  | fi-fj>Δf, ∀i≠j | (1) |
| --- | --- | --- |

The signal integration performed by the detector is confined to a finite time window T. This process is equivalent to multiplying the infinitely long original signal xt by a rectangular window function rectt/T of width T:

|  | xTt=xt⋅recttT | (2) |
| --- | --- | --- |

Assuming the original signal is a single-frequency cosine wave xt=cos2πf0t (whose spectrum is the impulse function δf-f0), the spectrum XTf of the truncated signal can broaden into a sinc function morphology:

|  | XTf=Xf*FrecttT=Xf*T⋅sincπfT | (3) |
| --- | --- | --- |

The derivation above demonstrates that the ideally sharp impulse spectral line broadens due to spectral leakage. The main lobe width of the sinc function (i.e., the Rayleigh bandwidth) directly determines the system's frequency resolving capability.

- Harmonic interference avoidance condition

Due to the hardware limitations discussed above, the j-th channel signal contains not only the fundamental frequency fj but also higher-order harmonics such as 2fj, 3fj, etc. There are two strategies to avoid harmonic interference: sub-octave design and specific frequency avoidance. If the frequency set satisfies the condition that the maximum frequency fmax is less than twice the minimum frequency fmin:

|  | fmax<2⋅fmin | (4) |
| --- | --- | --- |

Then, the second harmonic of the lowest frequency signal (2fmin) will reside above the highest frequency signal (fmax). In this sub-octave strategy, the entire fundamental frequency band is free of harmonic interference. However, if the system bandwidth is restricted or the number of channels is large, resulting in fmax≥2fmin, the following constraint must be enforced to ensure that no fundamental frequency coincides with the harmonics of other channels:

|  | fj≠n⋅fi, ∀i≠j, n∈{2,3,…,Nmax} | (5) |
| --- | --- | --- |

where Nmax is the highest harmonic order limited by the effective bandwidth fbw of the system.

- Intermodulation interference avoidance

|  | fi-fj≠fk, ∀i≠j≠k | (6) |
| --- | --- | --- |

Assuming the output response Isensort of the sensor contains a quadratic non-linear term:

|  | Isensort∝k=1KEkt2 | (7) |
| --- | --- | --- |

where Ekt=E0kcos2πfkt is the equivalent signal component of the k-th modulation kernel. Expanding the squared term yields:

|  | Isensort∝k=1KE0k2cos22πfkt+i≠jE0iE0jcos2πfitcos2πfjt | (8) |
| --- | --- | --- |

The cross-term can be expanded as:

|  | cos2πfitcos2πfjt=12cos2πfi+fjt+cos2πfi-fjt | (9) |
| --- | --- | --- |

This equation reveals that hardware non-linearities inevitably generate sum and difference frequency interferences. While sub-octave strategies inherently exclude sum-frequencies from the operational band, difference-frequencies—and both components in wider-band systems—can still cause contamination. If any combination frequency coincides with a target demodulation frequency fk, an uncalibratable interference term corrupts the measured amplitude:

|  | Ak=Ak+α⋅AiAj | (10) |
| --- | --- | --- |

Here, α represents the intermodulation coefficient and AiAj denotes the mixing contribution, which introduces non-linear distortion that fundamentally degrades spectral reconstruction.

Section 3: Frequency selection strategies

To eliminate spectral leakage caused by non-integer period truncation, the system satisfies the coherent sampling condition, requiring all modulation frequencies fk to be integer multiples of the frequency resolution Δf:

|  | fk=M⋅Δf, M∈Z+ | (11) |
| --- | --- | --- |

Guided by this fundamental constraint, we formulate two theoretical frequency selection strategies catering to different system bandwidth capacities and computational priorities.

1. Strategy 1: Linear planning based on sub-octave rules

Under the sub-octave constraint (fmax<2⋅fmin) second and higher-order harmonics of any channel inherently fall outside the effective signal band fmin,fmax condition eliminates the need for complex combinatorial optimization; orthogonality is guaranteed simply by spacing the frequencies linearly according to Eq. (11).

2. Strategy 2: Broadband optimization via greedy algorithm

When the system bandwidth is wide (fmax≥2⋅fmin), frequency selection becomes a multi-constraint combinatorial optimization problem. We design a greedy algorithm (Algorithm S2) as follows:

Algorithm S2: Greedy optimization for modulation frequency selection

Input：Lower bound fmin, upper bound fmax, resolution Δf, channel count K

Output：Frequency set H={f1,f2,…,fK}

1：　H←∅

2：　for f = fmin to fmax step Δf do

3： 　　if ∀f∈H，the candidate f satisfies:

4： 　　  fi-fj>Δf

5： 　　  f≠n⋅fi，∀n∈Z≥2

6： 　　  f-fi≠fj，∀f∈H

7： 　　  f+fi≠fj，∀f∈H

8： 　　then

9： 　　  H←H∪f

10： 　　 if H=K then return H

11：  end for

12：  return H

In this study, the practical hardware imposes a physical modulation bandwidth limit of 500 kHz. To evaluate the orthogonal frequency encoding (OFE) framework under this constraint, we apply the above frequency selection strategies across our numerical simulations and physical experiments.

Simulation setup (Strategy 1): The objective of the numerical simulation is to demonstrate the advantage of the OFE framework over conventional TDM in terms of noise rejection. We simply apply Strategy 1 as a representative interference-free configuration. By setting the lower bound fmin=380z and the frequency spacing Δf = 10 kHz, we establish the following 12-channel frequency set Hsub (in kHz):

|  | Hsub={380,390,400,410,420,430,440,450,460,470,480,490} | (12) |
| --- | --- | --- |

Experimental setup (Strategies 1 and 2): The hardware experiments are specifically designed to verify the practical feasibility of both frequency selection strategies in a real-world optoelectronic system. First, we deploy the sub-octave set Hsub (Strategy 1) to confirm that a linearly spaced, high-frequency configuration avoids harmonic and intermodulation distortions. Second, to validate the effectiveness of the greedy algorithm in a wideband scenario (fmax≥2⋅fmin), we implement Strategy 2. The optimal 12-channel broadband set Hopt (in kHz) generated and experimentally verified is:

|  | Hopt={30,50,70,110,130,170,190,230,290,310,370,410} | (13) |
| --- | --- | --- |

Section 4: Additive white gaussian noise (AWGN) model

In this study, noise interferences are treated as additive noise. The actual observed signal yt received by the system is modeled as a linear superposition of the ideal signal yt and the specific environmental noise nt applied independently:

|  | yt=yt+nt | (14) |
| --- | --- | --- |

Where s(t) is the noiseless signal (FDM or TDM) and nt∼N0,σn2 is independent and identically distributed Gaussian white noise. The noise variance is determined by a prescribed signal-to-noise ratio (SNR) in decibels:

σn2=Ps10SNRdB/10

where Ps is the signal power of the noiseless FDM waveform, computed as the temporal variance of the composite sinusoidal signal. To ensure a fair comparison, the same noise power σn2 and the same random realization are applied to both the FDM and TDM signals.

Section 5: Phase noise simulation

Beyond additive noise, phase instability in oscillator, typically arising from signal generators or phase-locked loops (PLLs), affects OTFE performance. The modulated signal with phase noise is modeled as:

snoisyt=k=1KAkcos2πfkt+ϕk+θkt

where θkt represents the phase jitter, modeled as a zero-mean Gaussian random process: θkt∼N0,σϕ2.

For a small phase deviation (σϕ≪1 rad), using the small-angle approximation (cosθ≈1,sinθ≈ θ), the signal for the k-th channel can be expanded as:

skt=Akcos2πfkt+ϕkcosθkt-Aksin2πfkt+ϕksinθkt
≈Akcos2πfkt+ϕkIdeal Carrier-Akθktsin2πfkt+ϕkInterference

This derivation reveals that phase noise introduces an orthogonal interference component modulated by the random process θkt. This results in spectral leakage (energy loss at the carrier frequency) and disrupts orthogonality, leading to inter-channel crosstalk.

Figure S1. Simulation of OTFE robustness against phase noise. (a) Time-domain waveform response under varying phase noise intensities (σϕ=0∼0.1 rad). The inset highlights that phase instability causes minor horizontal shifts in peak positions rather than amplitude distortion. (b) Spectral reconstruction RMSE as a function of phase noise standard deviation. The system demonstrates exceptional stability, with negligible error growth within the typical operating range of hardware oscillators (σϕ<0.01 rad).

Figure S1 illustrates the system's response to phase noise with standard deviations $\sigma_\phi$ ranging from 0 to 0.1 rad. As shown in Figure S1(a), unlike additive noise which induces sharp amplitude spikes or "burrs," phase noise manifests primarily as minute horizontal timing jitters (time-axis deviations), while the overall signal envelope remains smooth and intact.

Figure S1(b) quantifies the robustness. The spectral reconstruction error (RMSE) exhibits a slow linear increase, rising only marginally from 0.0085 to ≈0.01 even when σϕ reaches 0.1 rad (≈5.7∘). Given that modern commercial signal generators typically maintain phase noise well below 0.01 rad, the system operates in the flat region of the error curve, rendering oscillator instability negligible for practical spectral reconstruction.

Section 6: Definition of demodulation error

Figure 2b illustrates the correlation between spectral reconstruction RMSE and demodulation error. We define the demodulation error ηdemod as the relative Euclidean distance between the estimated measurement vector z and the ground truth vector z:

ηdemod=z-z2z2×100

This metric quantifies the efficiency of carrier extraction and noise rejection in the frequency domain.

Section 7: Spectral distribution of illumination sources

The system employs a discrete array of narrowband active illumination sources to span the visible spectrum. The spectral power distribution (SPD) of the k-th source, denoted as Pkλ, is modeled using a Gaussian approximation:

Pkλ≈Ppeak,k⋅exp-λ-λc,k22σk2

where λc,k represents the center wavelength, σk determines the spectral bandwidth, and Ppeak,k denotes the peak radiant intensity. These profiles form the basis vectors of the system matrix W used in the forward model (Equation 7 in the main text).

Figure S2 characterizes the spectral properties of the 12 discrete sources utilized in the prototype. The left panel illustrates the normalized spectral profiles (amplitude scaled to [0, 1]). The right panel displays the measured relative radiant flux of the sources under identical driving conditions. The variations in peak amplitude reflect the inherent differences in quantum efficiency and electro-optical conversion characteristics of different semiconductor materials across the spectrum. These intensity disparities are pre-calibrated and compensated for within the system matrix W.

Figure S2. Spectral characterization of the illumination sources. (a) Normalized spectral profiles of the 12 channels. The sources exhibit narrowband characteristics with distinct center wavelengths distributed across the visible band (400–700 nm). (b) Actual relative intensity distribution. The plot reveals the native radiant flux variations among the sources, which are accounted for in the system calibration matrix to ensure accurate radiometric retrieval.

Section 8: Definition of signal-to-noise ratio (SNR) in the frequency domain

To quantitatively assess the robustness of the orthogonal frequency encoding (OFE) framework, the signal-to-noise ratio (SNR) is defined in the frequency domain, consistent with our signal retrieval process. This definition accounts for practical hardware constraints and ensures a rigorous evaluation of the system’s noise rejection capabilities.

- Signal Power (Psignal)

In the experimental setup, the modulation signals generated by the hardware (e.g., LED drivers) are not ideal sinusoids and contain both even and odd harmonics. However, the spectral reconstruction algorithm exclusively extracts energy at the fundamental carrier frequencies {fk}. Consequently, the signal power is defined as the sum of the squared magnitudes at these specific frequencies:

Psignal=k=1KYfk2

where Yfk represents the complex magnitude of the spectrum at carrier frequency fk. All higher-order harmonics (nfk for n≥2) are excluded from Psignal as they do not contribute to the spectral reconstruction.

- Noise Power (Pnoise)

The noise power is defined as the total residual energy within the experimental bandwidth that could potentially interfere with the carrier extraction. To provide a conservative and precise assessment of external noise immunity, Pnoise is calculated by excluding the DC component, the fundamental carriers, and their associated higher-order harmonics:

Pnoise= f∈0,B,f∉nfkn≥1Yf2

where B denotes the maximum frequency of interest is set to 500 kHz, determined by the physical modulation limits of the system. The direct current (DC) at 0 Hz, the fundamental carriers {fk}, and all their higher-order harmonics {nfk | n=2,3,…} are strictly removed from the summation. By excluding harmonics from Pnoise, we treat these deterministic hardware artifacts as systematic losses rather than stochastic noise. Consequently, Pnoise focuses on environmental disturbances such as SMPS switching ripples and PWM dimming artifacts.

- SNR Calculation

The SNR is expressed as the ratio of the fundamental carrier power to the total residual interference power within the sensing bandwidth:

SNRdB=PsignalPnoise=10log10k=1KYfk2f∈0,B,f∉nfkn≥1Yf2

This frequency-domain definition provides a conservative estimate of the system's performance. By treating harmonic distortions as a form of self-interference and excluding them from the signal power, we ensure that the reported SNR (e.g., -11.77 dB) accurately reflects the framework's ability to recover high-fidelity spectral information from a heavily submerged signal. This approach maintains strict consistency between the SNR calculation and the actual physical mechanism of spectral retrieval.